BAO CAO DO AN IOT SMART HOME

Mo phong Wokwi ESP32 - MQTT - Firebase - ReactJS

| Hang muc | Noi dung |
| --- | --- |
| De tai | He thong Smart Home doc cam bien va dieu khien den chinh |
| Nen tang phan cung | ESP32 mo phong tren Wokwi |
| Backend | Node.js Express, MQTT client, Firebase Admin SDK |
| Frontend | ReactJS + TypeScript, Firebase Realtime Database |
| Ban cap nhat | Chi giu Manual control cho lamp GPIO2; LED canh bao tu dong |

Tom tat thay doi kien truc Manual Mode khong con dieu khien rainLed, motionLed, gasLed, tempLed. Bon LED nay la LED canh bao va phai tu dong bam theo cam bien. Dashboard chi con dieu khien lamp chinh GPIO2 khi mode = manual.

# 1. Muc tieu he thong

Mo phong mot nha thong minh co doc nhiet do, do am, gas, anh sang, mua va chuyen dong.

Gui du lieu cam bien tu ESP32 len MQTT, backend nhan MQTT va ghi vao Firebase.

Dashboard React lang nghe Firebase realtime de hien thi thong so va trang thai canh bao.

Cho phep nguoi dung chuyen Auto/Manual va dieu khien rieng den chinh GPIO2.

Dam bao LED canh bao luon phan anh dung trang thai cam bien, khong bi nguoi dung tat nham.

# 2. Kien truc tong the sau khi chinh

Luong realtime chinh Wokwi ESP32 -> MQTT broker -> Backend Node.js -> Firebase Realtime Database -> React Dashboard React Dashboard -> Backend REST API -> MQTT broker -> Wokwi ESP32

| Lop | Thanh phan | Vai tro |
| --- | --- | --- |
| Perception | DHT22, MQ-2, LDR, rain button, PIR | Tao du lieu moi truong va trang thai su kien |
| Device | ESP32 | Doc cam bien, dieu khien GPIO, publish/subscribe MQTT |
| Network | MQTT broker HiveMQ | Truyen message giua ESP32 va backend |
| Middleware | Node.js backend | Nhan MQTT, ghi Firebase, nhan lenh REST tu dashboard |
| Data/UI | Firebase + React Dashboard | Luu trang thai realtime va hien thi giao dien |

# 3. Phan cung Wokwi va mapping chan

| Thiet bi | GPIO | Kieu du lieu | Y nghia |
| --- | --- | --- | --- |
| DHT22 SDA | GPIO4 | Digital | Nhiet do va do am |
| MQ-2 AOUT | GPIO34 | ADC | Gia tri gas |
| LDR AO | GPIO35 | ADC | Cuong do anh sang |
| Rain button | GPIO18 | Input pullup | Mo phong trang thai mua |
| PIR OUT | GPIO19 | Digital | Phat hien chuyen dong |
| Lamp LED | GPIO2 | Output | Den chinh, Auto theo LDR hoac Manual tu dashboard |
| Rain LED | GPIO5 | Output | Canh bao mua, tu dong theo rain |
| Motion LED | GPIO16 | Output | Canh bao chuyen dong, tu dong theo PIR |
| Gas LED | GPIO17 | Output | Canh bao gas, tu dong theo nguong gas |
| Temp LED | GPIO21 | Output | Canh bao nhiet, tu dong theo nguong nhiet |

# 4. Quy tac Auto va Manual

| Trang thai | Den chinh GPIO2 | LED canh bao |
| --- | --- | --- |
| Auto | ESP32 tu bat khi light < 2000 ADC, tu tat khi du sang | Luon tu dong theo cam bien |
| Manual | Dashboard duoc gui lenh lamp = 0/1 qua backend | Van tu dong theo cam bien, khong cho dieu khien tay |

Ly do bo dieu khien thu cong LED canh bao Rain LED, Motion LED, Gas LED va Temp LED co y nghia canh bao. Neu cho nguoi dung tat thu cong, dashboard co the bao sai thuc te. Do do cac LED nay phai duoc tinh tu sensor tai ESP32.

# 5. MQTT topics, Firebase paths va API

| Kenh | Ten | Huong | Noi dung |
| --- | --- | --- | --- |
| MQTT | smarthome/sensors/data | ESP32 -> Backend | temperature, humidity, gas, light, rain, motion, lamp, warning, mode |
| MQTT | smarthome/devices/status | ESP32 -> Backend | lamp, rainLed, motionLed, gasLed, tempLed, mode |
| MQTT | smarthome/mode/control | Backend -> ESP32 | { mode: auto ¦ manual } |
| MQTT | smarthome/devices/control | Backend -> ESP32 | { lamp: 0 ¦ 1 } |
| Firebase | smarthome/current | Backend -> Firebase -> Web | Du lieu cam bien hien tai |
| Firebase | smarthome/devices/status | Backend -> Firebase -> Web | Trang thai LED va mode |
| API | POST /api/mode | Web -> Backend | Doi Auto/Manual |
| API | POST /api/control | Web -> Backend | Chi nhan field lamp |

# 6. Thay doi trong source code

| File | Noi dung da chinh |
| --- | --- |
| frontend/src/lib/types.ts | DeviceControl chi con lamp?: number |
| frontend/src/components/SmartHomeDashboard.tsx | ControlPanel chi hien nut Den chinh GPIO2; mode lay tu device status |
| backend/src/routes/controlRoutes.js | POST /api/control chi chap nhan lamp va yeu cau mode manual |
| backend/src/firebase.js | Them getDeviceStatus va updateDeviceStatus de UI cap nhat nhanh |
| frontend/esp32/smart_home.ino | Manual chi xu ly lamp; LED canh bao sync tu cam bien trong moi mode |

# 7. NV3 - Demo mot vai duong di cua luong du lieu

Demo 1: Gas vuot nguong MQ-2 AOUT -> ESP32 analogRead(GPIO34) gasValue > 2000 -> gasWarning = 1 -> gasLed = 1 ESP32 publish smarthome/sensors/data va smarthome/devices/status Backend subscribe MQTT -> saveCurrentData() va saveDeviceStatus() Firebase smarthome/current + smarthome/devices/status thay doi React subscribeToTelemetry() -> Metric Gas doi mau canh bao, Tong canh bao = Dang co canh bao

Demo 2: Nhan nut mua tren Wokwi Rain button GPIO18 duoc nhan -> rainStatus toggle ESP32 dat rain = 1 va rainLed = 1 ESP32 publish sensors/status qua MQTT Backend ghi Firebase -> Dashboard StatusTile Nut rain hien Dang mua

Demo 3: Dieu khien den chinh tu dashboard Nguoi dung bam Manual -> POST /api/mode { mode: 'manual' } Backend publish smarthome/mode/control -> ESP32 set currentMode = manual Nguoi dung bam Den chinh GPIO2 -> POST /api/control { lamp: 1 } Backend publish smarthome/devices/control -> ESP32 digitalWrite(GPIO2, HIGH) ESP32 publish devices/status -> Firebase -> Dashboard hien Den chinh ON

Demo 4: Auto bat den theo LDR LDR AO -> ESP32 analogRead(GPIO35) Neu lightValue < 2000 va currentMode = auto -> lamp = 1 ESP32 ghi GPIO2 HIGH va publish status Dashboard nhan status tu Firebase va hien Den chinh Dang bat

# 8. Ket luan

Kien truc moi dung y nghia dieu khien: nguoi dung chi dieu khien den chinh.

Cac LED canh bao duoc bao ve khoi thao tac thu cong va luon phan anh cam bien.

Backend dong vai tro bridge MQTT - Firebase - REST API ro rang hon.

Dashboard gon hon, tranh gay nham lan giua den canh bao va den nguoi dung dieu khien.